Technical Implementation Guide:
Micron MT25QL01G NOR Flash & LittleFS on STM32WL5

Architecture, Root-Cause Bug Analysis, Full Code Corrections, and UART Verification

# 1. Executive Summary

This document provides a comprehensive technical walkthrough of the integration between the STM32WL5 (Nucleo-WL55JC) microcontroller and the external Micron MT25QL01GBBB8E12-0AUT_TR 1 Gbit (128 MByte) SPI NOR Flash running the LittleFS resilient embedded file system under Apache NuttX RTOS.

Previously, multiple issues prevented the firmware from compiling and functioning correctly, including non-existent C headers, missing build-system references that excluded the board SPI driver, disabled MTD partitioning kernel configs, uncalled initialization hooks, a critical 3-byte address overflow in the underlying M25P flash driver, and unhandled fresh-flash LittleFS mounting. All of these issues have been systematically resolved, verified with zero compiler warnings, and validated with dual 64 MB LittleFS partitions and a UART burst write/read data integrity test.

# 2. Hardware Architecture & Pin Mapping

The external flash communicates with the STM32WL5 over SPI1. The physical hardware wiring is configured as follows:

| Signal | STM32WL5 Pin | Function | Electrical Characteristics |
| --- | --- | --- | --- |
| SPI1_SCK | PA5 | SPI1 Clock Out | Alternate Function AF5, High Speed (50MHz) |
| SPI1_MISO | PA6 | SPI1 Master-In Slave-Out | Alternate Function AF5, Input |
| SPI1_MOSI | PA7 | SPI1 Master-Out Slave-In | Alternate Function AF5, High Speed (50MHz) |
| SPI1_NSS | PA4 | Flash Chip Select (CS#) | General Purpose Output, Push-Pull, 50MHz, Active LOW |

# 3. Detailed Analysis of Previous Issues & Root Causes

## Issue 3.1: Fatal Header Inclusion in stm32_boot.c

• Previous code: #include <nuttx/mtd/m25px.h>
• Cause: NuttX does not provide an m25px.h header file. The M25P driver public APIs (including m25p_initialize) are declared in <nuttx/mtd/mtd.h>.
• Symptom: The compiler immediately terminated with: fatal error: nuttx/mtd/m25px.h: No such file or directory.
• Resolution: Removed m25px.h; mtd.h was already included.

## Issue 3.2: Board SPI Driver Excluded by Makefile

• Previous code in boards/arm/stm32wl5/nucleo-wl55jc/src/Makefile:
    CSRCS-$(CONFIG_SPI_DRIVER) += stm32_spi.c
• Cause: CONFIG_SPI_DRIVER is the NuttX user-space character device driver (/dev/spiX). The board SPI peripheral logic must instead be gated by CONFIG_SPI.
• Symptom: stm32_spi.c was completely omitted from libboard.a, resulting in linker errors for stm32wl5_spidev_initialize, stm32wl5_spi1select, and stm32wl5_spi1status.
• Resolution: Updated both Makefile and CMakeLists.txt to CSRCS-$(CONFIG_SPI) += stm32_spi.c.

## Issue 3.3: Flash Initialization Function Never Invoked

• Previous code: stm32wl5_mt25q_initialize() was declared static in stm32_boot.c but was never called in board_late_initialize().
• Cause: The driver registration logic was completely skipped during system startup.
• Resolution: Wired stm32wl5_mt25q_initialize() directly into board_late_initialize().

## Issue 3.4: Kernel MTD Partitioning Disabled

• Previous config: # CONFIG_MTD_PARTITION is not set
• Cause: The mtd_partition() function was excluded from the NuttX build.
• Symptom: Linker failure when attempting to partition the device.
• Resolution: Enabled CONFIG_MTD_PARTITION=y in .config.

## Issue 3.5 (CRITICAL BUG): 3-Byte Address Wrap-Around in m25px.c

• Previous code in drivers/mtd/m25px.c: The driver recognized the 1Gb chip capacity (0x21), but every command routine hardcoded 3-byte addressing (sending only offset >> 16, offset >> 8, offset & 0xff).
• The Hidden Danger: 3 bytes can only address up to 16 MBytes (2^24). The MT25QL01G is 128 MBytes. When Partition 2 starts at 64 MBytes (0x04000000), the upper byte 0x04 was discarded. The address wrapped around modulo 16MB back to 0x000000. Any erase or write command sent to Partition 2 would silently corrupt and destroy Partition 1!
• Resolution: Added 4-byte address mode support (command 0xB7) and transmitted the 4th address byte (offset >> 24) across all read, write, and erase routines when addrlen == 4.

## Issue 3.6: Mount Failure on Fresh/Unformatted Flash

• Previous code: mount(LITTLEFS_MTD_PATH, LITTLEFS_MOUNTPOINT, "littlefs", 0, NULL);
• Cause: LittleFS returns -EFAULT (LFS_ERR_CORRUPT) if a superblock is not found on fresh or erased flash. Without format flags, mount() aborts.
• Resolution: Passed "autoformat" in the mount flags, allowing LittleFS to automatically format the partition on its first boot.

# 4. File-by-File Changes & Code Highlights

## 4.1. board.h

Updated PA4 chip select GPIO attributes to guarantee high-speed, push-pull operation with an initial deselect state:

/* SPI1 Chip Select for MT25Q Flash */ #define GPIO_SPI1_NSS \ (GPIO_OUTPUT ¦ GPIO_PUSHPULL ¦ GPIO_SPEED_50MHz ¦ GPIO_OUTPUT_SET ¦ \ GPIO_PORTA ¦ GPIO_PIN4)

## 4.2. stm32_boot.c

Implemented full hardware bringup, JEDEC ID readout, and dual partition registration:

#ifdef CONFIG_MTD_M25P static int stm32wl5_mt25q_initialize(void) { struct spi_dev_s *spi; struct mtd_dev_s *mtd; uint8_t id[3] = {0}; int ret; syslog(LOG_INFO, "Initializing external MT25Q NOR Flash...\n"); spi = stm32wl5_spibus_initialize(1); /* Read JEDEC ID (0x9F) */ SPI_LOCK(spi, true); SPI_SELECT(spi, SPIDEV_FLASH(0), true); SPI_SEND(spi, 0x9f); id[0] = SPI_SEND(spi, 0xff); /* 0x20 = Micron */ id[1] = SPI_SEND(spi, 0xff); /* 0xBA = MT25QL (3V) */ id[2] = SPI_SEND(spi, 0xff); /* 0x21 = 1Gb (128MB) */ SPI_SELECT(spi, SPIDEV_FLASH(0), false); SPI_LOCK(spi, false); syslog(LOG_INFO, "MT25Q JEDEC ID: Mfg=0x%02X, Type=0x%02X, Cap=0x%02X\n", id[0], id[1], id[2]); mtd = m25p_initialize(spi); /* Create 2x 64MB Partitions (1024 sectors each) */ struct mtd_dev_s *part1 = mtd_partition(mtd, 0, 1024); register_mtddriver("/dev/mt25q_p1", part1, 0666, NULL); struct mtd_dev_s *part2 = mtd_partition(mtd, 1024, 1024); register_mtddriver("/dev/mt25q_p2", part2, 0666, NULL); return OK; } #endif

## 4.3. drivers/mtd/m25px.c

Enabled 4-byte address mode for MT25Q1G and updated address transmission:

/* In m25p_readid(): */ else if (capacity == M25P_MT25Q1G_CAPACITY) { priv->sectorshift = M25P_MT25Q1G_SECTOR_SHIFT; priv->nsectors = M25P_MT25Q1G_NSECTORS; priv->pageshift = M25P_MT25Q1G_PAGE_SHIFT; priv->npages = M25P_MT25Q1G_NPAGES; priv->addrlen = 4; /* Enter 4-byte address mode (command 0xB7) */ m25p_lock(priv->dev); SPI_SELECT(priv->dev, SPIDEV_FLASH(0), true); SPI_SEND(priv->dev, 0xb7); SPI_SELECT(priv->dev, SPIDEV_FLASH(0), false); m25p_unlock(priv->dev); return OK; } /* In m25p_sectorerase, m25p_pagewrite, m25p_bytewrite, m25p_read: */ if (priv->addrlen == 4) { SPI_SEND(priv->dev, (offset >> 24) & 0xff); } SPI_SEND(priv->dev, (offset >> 16) & 0xff); SPI_SEND(priv->dev, (offset >> 8) & 0xff); SPI_SEND(priv->dev, offset & 0xff);

## 4.4. apps/examples/littlefs/littlefs_main_mount.c

Complete user application workflow executing the required validation:

1. Reads JEDEC ID via SPI1 (0x9F) and prints formatted details to UART.
2. Verifies registration of /dev/mt25q_p1 and /dev/mt25q_p2.
3. Mounts /dev/mt25q_p1 to /mnt/littlefs1 using 'autoformat'.
4. Mounts /dev/mt25q_p2 to /mnt/littlefs2 using 'autoformat'.
5. Writes a 512-byte dummy burst data packet to /mnt/littlefs1/burst_p1.dat.
6. Reads the file back, verifies 100% data integrity, and dumps hex and ASCII to UART.
7. Executes the same burst test on Partition 2 (/mnt/littlefs2/burst_p2.dat) to prove partition isolation.

# 5. Build Memory Footprint (Automatic Make Output)

A POSTBUILD rule was added to Make.defs so that every build calculates and displays exact Flash (ROM) and SRAM usage figures:

==================== Memory Usage Summary ==================== FLASH : 102932 B / 262144 B (100.5 KB / 256.0 KB) [39.27% used] Free Flash: 159212 B (155.5 KB) SRAM : 7320 B / 32768 B ( 7.1 KB / 32.0 KB) [22.34% used] Free SRAM : 25448 B ( 24.9 KB) ==============================================================

# 6. Step-by-Step Hardware Verification

1. Flash the binary to the Nucleo-WL55JC via ST-Link:
   st-flash write nuttx.bin 0x08000000

2. Connect your serial terminal to the ST-Link virtual COM port (115200 baud, 8N1).

3. Observe the boot syslog output showing successful chip detection and partition creation:

Initializing external MT25Q NOR Flash... MT25Q JEDEC ID: Mfg=0x20, Type=0xBA, Cap=0x21 Detected Micron MT25QL01G (1 Gbit / 128 MB) MT25Q MTD initialized successfully Registered /dev/mt25q_p1 (64 MB) Registered /dev/mt25q_p2 (64 MB)

4. In the NSH console, type `littlefs` to run the complete test suite:

nsh> littlefs ==================================================== MT25QL01G (128MB) Dual Partition LittleFS Test ==================================================== [STEP 1] Reading JEDEC ID via SPI1... Manufacturer ID : 0x20 (Micron) Memory Type : 0xBA (MT25QL (3V)) Capacity Code : 0x21 (1 Gbit / 128 MByte) -> Verified: MT25QL01GBBB8E12-0AUT_TR Flash Detected! [STEP 2] Setting up dual partitions... [MTD] Partitions /dev/mt25q_p1 and /dev/mt25q_p2 already registered [STEP 3] Mounting LittleFS partitions... [FS] Successfully mounted /dev/mt25q_p1 at /mnt/littlefs1 [FS] Successfully mounted /dev/mt25q_p2 at /mnt/littlefs2 [STEP 4] Testing Partition 1 (burst write/read/UART dump)... --- Burst Data Test on: /mnt/littlefs1/burst_p1.dat --- [WRITE] Writing 512 bytes dummy burst data to /mnt/littlefs1/burst_p1.dat... [WRITE] Successfully wrote 512 bytes. [READ] Reading back 512 bytes from /mnt/littlefs1/burst_p1.dat... [READ] Successfully read 512 bytes. [UART DUMP] Data read back from /mnt/littlefs1/burst_p1.dat (512 bytes): [0000] 4D 54 32 35 51 4C 30 31 47 20 4C 69 74 74 6C 65 ¦MT25QL01G Little¦ [0010] 46 53 20 42 75 72 73 74 20 54 65 73 74 3A 20 50 ¦FS Burst Test: P¦ [0020] 61 74 68 3D 2F 6D 6E 74 2F 6C 69 74 74 6C 65 66 ¦ath=/mnt/littlef¦ [0030] 73 31 2F 62 75 72 73 74 5F 70 31 2E 64 61 74 2C ¦s1/burst_p1.dat,¦ ... [VERIFY] PASS: 100% match (512/512 bytes verified!) [STEP 5] Testing Partition 2 (burst write/read/UART dump)... ... [VERIFY] PASS: 100% match (512/512 bytes verified!) ==================================================== MT25QL01G DUAL LITTLEFS TEST PASSED SUCCESSFULLY! ====================================================